ROMEXIS PMBRIDGE

V.6.1.0

SPECIFICATION

1.	ROMEXIS PMBRIDGE Interface Specification	5

1.1	Background	5

1.2	The big picture	6

1.3	Planmeca implementation of PMBRIDGE for ROMEXIS	7

2.	Using the PMBRIDGE	7

2.1	Library	7

2.2	Functions	7

3.	PMBRIDGE Functions	8

3.1	Session Related Functions	8

3.1.1	At the beginning of the session	8

3.1.2	Terminating the session	8

3.1.3	Application window behavior	8

3.1.4	Set location of the main window	9

3.1.5	Force Romexis to be on top	9

3.1.6	Hide Toolbar & Statusbar of ROMEXIS	9

3.1.7	Define Limits for the ROMEXIS Window	9

3.1.8	Select Romexis Module	10

3.1.9	Get Clinic Status	10

3.2	Image Related Functions	10

3.2.1	Radiograph request	10

3.2.2	Opening the image acquisition dialog	11

3.2.3	Opening the image acquisition dialog with study option	11

3.2.4	Opening the image selection dialog or given image	11

3.2.5	Getting information about images of the current patient	13

3.2.6	Setting Image Retrieval Window Size	13

3.2.7	Receiving of the image data	13

3.2.8	Getting a thumbnail image	14

3.2.9	Inquire CAD/CAM Case(s)	15

3.2.10	Open/Launch CAD/CAM Case	16

3.2.11	Get Image Diagnose	16

3.2.12	Set Image Diagnose	17

3.3	User and Patient Related Functions	17

3.3.1	Adding a new patient	17

3.3.1	Querying patient information	18

3.3.2	Modifying patient information	18

3.3.3	Changing patient ID	18

3.3.4	Removing patient	19

3.3.5	Deselection of the current patient	19

3.3.6	Setting ROMEXIS User	19

3.4	Study Related Functions	19

3.4.1	Get Number of Studies	20

3.4.2	Get Study IDs	20

3.4.3	Get Study IDs and ‘Names’	21

3.4.4	Get Number of Images in Study	21

3.4.5	Get Study Info	21

3.4.6	Start Study Capture	21

4.	PmBridge / Romexis Patient Information Fields	22

4.1	PatientID	22

4.2	FamilyName	22

4.3	FirstName	22

4.4	Birthdate	22

4.5	Doctor(optional)	22

5.	Best practices	22

5.1	Opening patients	22

5.2	High Level Bridge Command Line Application	23

5.3	Using DxStart() Function	23

6.	Installation	23

6.1	Windows Operating System	23

6.2	Macintosh Operating System	24

6.3	Conflicts with DIMAXIS PMBRIDGE	24

7.	Other	24

7.1	Example Program DxClientW	24

7.2	Unicode Compatibility	24

7.3	Log File	25

7.4	Clinic Status XML Sample	26

7.5	Real Basic PmBridge Interface Declarations Samples	26

romexis PMbridge 6.1.0 SpeCifiCation

Planmeca reserves the right to make changes to this specification without a further notice. Planmeca disclaims any liability, including without limitation consequential or incidental damages related to the use the contents of this document. There are no express or implied copyrights or patent licenses granted hereunder to copyrighted material or applicable patents of Planmeca or others.

Having said that, we hope the information presented here will be useful and we will proceed to improve both the documentation and the PMBRIDGE standard it describers, as well as other support material for the PMBRIDGE interface. All comments and suggestions are highly appreciated -- we hope to establish open and productive relationships with third party software developers as well as other dental imaging device manufactures.

Please send comments etc to:

Aftersales

Planmeca Oy

Asentajakatu 6

00810 HELSINKI

FINLAND

Or

Email: aftersales@planmeca.com

# ROMEXIS PMBRIDGE Interface Specification

This document describes the PMBRIDGE interface for PLANMECA ROMEXIS imaging software. Although it has been defined exclusively by Planmeca Group, it is an open system that we hope will be widely used by third party software developers and dental imaging device manufacturers.

The version number of the PMBRIDGE interface refers to current ROMEXIS version number at the time that the interface has been modified. Possible modifications will always be done so that the library will remain compatible with older versions. All the functions presented in this document refer to this version of the PMBRIDGE.

Note thatthere are separate files for Windows and macOS versions of PMRIDGE for ROMEXIS. They are delivered on the installation media’s of the respected products and are not compatible with both imaging applications. 

Note also that the version numbers of the respective DLL files depend on the imaging software version number.

## Background

To be a successful part of a dental practice Planmeca has recognized the need for a way to "bridge" a practice management software with Planmeca Romexis imaging software. For this purpose Planmeca has defined a standard called PMBRIDGE and the supporting libraries and documentation.

## The big picture

The co-operation of the practice management software and the image manipulation software is illustrated in the following diagram.

Practice management

software

Planmeca Imaging Software 
(ROMEXIS) (review)rogram DxClient Romexisto the Romexis pmbridge ts to the Dimaxis me with PMBRIDGE. ed with Romexis imaging application.(review)rogram DxClient Romexisto the Romexis pmbridge ts to the Dimaxis me with PMBRIDGE. ed with Romexis imaging application.

USER

patient

&

practice

database

image

database

PMBRIDGE

patient ID,

control info

image

viewing, enhancement

interaction

patient selection

and

patient data

management

interaction

In this integration model the Planmeca Imaging Software will handle all the image manipulation and viewing related operations and user interactions. All the patient/practice data related operations (including patient selection) and interactions are to be handled by the practice management software. This system makes it very simple and quick for the practice management software house to create this link or bridge to the Planmeca Imaging Software. The actual coding time is measured in hours.

All that the practice management software has to do is to start the Planmeca Imaging Software (minimized) at start of the session. When the user clicks at a button (labeled e.g. "X-RAY"), the patient ID is passed to the imaging software by the PMBRIDGE library along with the request for the imaging software to show its windows. That's all. The imaging software will then display a dialog where all the patient images are listed and the user continues the interaction with the imaging software viewing, manipulating, printing etc.

The downside of this type of bridging is that it is not totally seamless which means that the user does see two programs with possibly different look-and-feel.

## Planmeca implementation of PMBRIDGE for ROMEXIS

The behavior of the ROMEXIS software is subtly changed when used via PMBRIDGE system: the splash screen at the launch of the application is omitted and the patient selection related functions are disabled as this will be handled by the practice management software via the bridge.

More information about PMBRIDGE and Planmeca ROMEXIS is available in Planmeca Romexis Technical manual.

# Using the PMBRIDGE

## Library

The PMBRIDGE is supplied in the form of a DLL library and associated '.H' and ‘.LIB’ / ‘.DYLIB’ files. Please note that the latest PmBridge.DLL / libPmBridge.dylib that come with ROMEXIS installation (Windows folder Romexis\Pmbridge\Program, MacOS folder /usr/local/lib) should always be used. The libraries may not be copied or moved to other locations, because otherwise the link will not work.

(Windows note! However sometimes an application cannot find the library and/or ‘Romexis.bat’ batch program, although the Windows system variable PATH contains a correct folder path. Then one needs to copy PmBridge.dll and Romexis.bat files into application work folder.)

In Windows it is recommended to use LoadLibrary() and GetProcAddress() calls for using PmBridge library instead of static linking with PmBridge.LIB. This way you can keep your program compatible with older PMBRIDGE versions which don’t have the new functions.

## Functions

All the functions return a result code that has either the value PMB_OK or signals some kind of abnormal condition.

A sample code, which can be used to simulate and investigate the practice management software side of bridging, is supplied with SDK for PMBRIDGE. Samples are written using MS Visual Studio 2015 for Windows and XCode for Mac OS X.

# PMBRIDGE Functions

## Session Related Functions

### At the beginning of the session

At the beginning of the session start the ROMEXIS software as follows:

err = DxStart();

Function contains finite loop for trying to start a Romexis session.

### Terminating the session

It is possible to terminate ROMEXIS by calling

err = DxExit ();

This could be done when the user is closing the administrative application. If ROMEXIS has modal dialog open, it can’t be terminated. In such a case the return code of DxExit() is PMB_DX_BUSY.

### Application window behavior

The practice management software may want to minimize the ROMEXIS application when the practice management software is switched to the foreground. This can be done with a call:

err = DxSetWindowPlacement (SW_SHOWMINIMIZED);

ROMEXIS can be configured to remember the window position after exiting the program. This feature is often useful when using ROMEXIS via PMBRIDGE. It is also possible to configure ROMEXIS to stay “Always on top”. This is useful for example when a dentist wants to write diagnose of an image in the practice management software while viewing the image in a ROMEXIS window.

Note! By default PmBridge Romexis.bat/Romexis.sh start script contains /PMBRIDGE keyword which sets Romexis window as minimized. It can be replaced by /PMBRIDGE_MAXIMIZED which sets window as maximized.

### Set location of the main window

It is possible to explicitly specify the location of the main window of Romexis by calling

DxMoveWindow (x0, y0, nWidth, nHeight );

where x0 and y0 specify the upper left corner of the window and nWidth and nHeight specify the size of the window.

### Force Romexis to be on top

Romexis can be forced to be always on top by calling

short nTop=1;

DxAlwaysOnTop (nTop);

### Define Limits for the ROMEXIS Window

Define limits for the Romexis window when the window is maximized. Using this function it is possible to make sure that the calling application's menu is visible when the window is resized. A value will not be used if it <= 0. The values are relative to the actual screen size. (This has been tested only in single monitor systems). Romexis also does a 'sanity' check for the values and uses value only if it does not limit screen usage more than 10%.

extern "C" short WINAPI DxSetMaxLimits (

long nMinTop,

long nMaxBottom,

long nMinLeft,

long nMaxRight

);

Note! It is possible to set default size of Romexis window in  Romexis.bat/Romexis.sh start script by adding parameters xsize=<value>  ysize=<value>

With function 
	extern "C" short WINAPI DxSetWorklistEntry ();
it is possible to pass to Romexis the same information as is used in DICOM Worklist. See PmBridge.h for details.

### Select Romexis Module

Change the active UI module (in PmBridge 4.5.0 or higher). All available module numbers defined in PmBridge.h header file.

extern "C" short  DxSelectRmxModule (

int nModule

);

Note! For some modules a patient must have been selected (use DxSelectPatient) before calling successfully this function.

### Get Clinic Status

Get PMU (Planmeca Unit network) status to a  XML file.

extern "C" short  DxGetClinicStatus(const char *pszFileName);

pszFileName    Input: Full path name of result XML file of char type

Get status XML to string

extern "C" short  DxGetClinicStatusString(WCHAR *wszStatus);

wsStatus       Input: Wide char array of max 20 480 characters

## Image Related Functions

### Radiograph request

When the user requests to see the radiographs of a particular patient, pass the patient information to ROMEXIS as follows:

// Set patient ID of PMS software of char type

char patientID[64];

sprintf(“%s”,64,patientID);

short noOfImages;

err = DxSelectPatient (patientID,&nNumberOfImages);

if (err==PM_OK)

Please note that the patient ID must exactly match the ID used in ROMEXIS. In ROMEXIS it is possible to specify minimum patient ID length. If the given ID is shorter than the specified minimum, ROMEXIS will add leading zeroes to the ID. Default value for minimum patient ID in ROMEXIS is 1, so no leading zeros will be added by default.

Immediately after that force ROMEXIS to show its window:

err = DxSetWindowPlacement(SW_SHOWNORMAL);

That's all that there is to the basic pattern of interaction.

### Opening the image acquisition dialog

Image acquisition dialog can be opened by calling DxCaptureImage(), for example

short nImageType = DX_IT_PAN ;

DxCaptureImage (nImageType);

### Opening the image acquisition dialog with study option

Image acquisition dialog with study selection window can be opened by calling DxCaptureImageIntra()

extern "C" short WINAPI DxCaptureImageIntra (
 short nUseStudy, short nAllowWithoutStudy
 );

nUseStudy ... 1 open Study Selection window, 0 don't use Study Selection window

nAllowWithoutStudy      ... 1 allow capture without Study, 0 don't allow capture without study

### Opening the image selection dialog or given image

A dialog for selecting images can be opened by calling

DxSelectImage ();

Note that there are several ways of selecting images in Romexis. The default method can be configured in Romexis Settings.

Planmeca application opens an 2D/3D image by calling

extern "C" short  DxOpenImage (

short nIndex,

int nImageID

nIndex   	Given index in image array

nImageID	Image ID

Following input combinations are handled

- the given index of image array (nIndex > -1 and nImageID < 1)

- the given image ID number (nImageID > 0)

- the given index and ID number (nIndex > -1 and nImageID >0)

In cases of 2) and 3) Planmeca application will check if given image ID really exists on Romexis database

### Getting information about images of the current patient

It is possible to get information about images of the current patient by calling

DxInquireImage(

short nIndex,            // 0 .. nImageCnt-1

long  arrImageParam [],  // Size must be at least DX_MAX_INDEX

short arrToothMask  []   // 1-based table for dentity

);

nIndex is the 0 –based index of the images of the patient. The number of the images the patient has can be received when calling DxSelectPatient()

arrImageParam[] is a 1-based array of the image parameters. The size of the array must be at least DX_MAX_INDEX. The indices of the table have been defined in “PmBridge.h”.

### Receiving of the image data

The image data can be received by calling

DxGetImage (

short nIndex,

short nConvertTo8bit,

short nEnhance,

short nFileFormat,

const char *pszFileName

);

nIndex 	see DxInquireImage()above

nConvertTo8bit is a flag. The x-ray images in Romexis are normally stored with 16(12) bits. If this flag is set to 1, the image will be mapped to 8 bits. Most applications are only able to handle 8 bit images.

nEnhance is a flag. If this flag is set to 1, the contrast of the image will be enhanced before saving the image to file. For cephalometric images the soft tissue filter will be applied. If the image has been explicitly saved, any other image processing functions (filtering etc) will be applied to the image.

nFileFormat defines the image file format; see “PmBridge.h” for definitions of the formats.

pszFileName is the output file name. It should include the extension (.TIF, .JPG or .BMP depending on nFileFormat).

or by calling

extern "C" short WINAPI DxGetImageByID (
 int nImageID,
 short nConvertTo8bit,
 short nEnhance,
 short nFileFormat,
 const char *pszFileName
 );

Otherwise the same as DxGetImage() except the nImageID is the real image number which is fetched from the database (with the function DxInquireImage() into the table arrImageParam[] with index value 10).

### Getting a thumbnail image

Besides image data, it is possible to get the thumbnail (overview) image by calling

extern "C" short WINAPI DxGetOverviewImage (

short nIndex,

short nFileFormat,

const char *pszFileName

);

See DxGetImage for definition of the parameters.

or by calling

extern "C" short WINAPI DxGetOverviewImageByID (
 int nImageID,
 short nFileFormat,
 const char *pszFileName
 );

Otherwise the same as DxGetOverviewImage() except the nImageID is the real image number which is fetched from the database with the function DxInquireImage() into the table arrImageParam[] with index value 10.

### Inquire CAD/CAM Case(s)

Implemented only for Romexis. A new function is added into 2.6.1 specification. (Romexis PmBridge library version 4.4.2 or greater)

It is possible to get information about cases of the current patient in XML format by calling

extern "C" short  DxInquireCadCamCase (

const int nID,

const char *pszFileName

);

nID 	Input:  ID number (via image info) for specific case,  or  number 0 for inquiring all cases

pszFileName  Full pathname for output XML –file.

Returns PMB_OK if case(s) found, or an error code PMB_CAMCAM_CASE_NOT_FOUND – when either any cases found or selected case not found.

XML file contains information about CAD/CAM case(s) and info about STL files a case contains. It is possible to get thumbnails and STL files using DxGetImageOverview and DxGetImage/byID functions.

Note 1. CAD/CAM case is a ZIPPED binary ‘image’ file, so requesting a thumbnail for a case using DxGetImageOverview() will return “NO PREVIEW” icon

Note 2. In image info array table (via DxInquireImage) returns a “CAD/CAM Case Image” info image type as type DX_IT_BINARY (i.e. an attachment) and image subtype as DX_IST_CADCAM_CASE

Note 3. Launch/open a CAD/CAM case via PmBridge DxOpenCadCase function!

### Open/Launch CAD/CAM Case

Implemented only for Romexis. A new function is added into 2.6.1 specification. (Romexis PmBridge library version 4.4.2 or greater)

extern "C" short  DxOpenCadCamCase (

const int nCaseID

);

nCaseID	Input:  case ID number. If number = 0, PlanCADprogram  is launched in “new case” mode

CAD/CAM case id number is parsed from XML file. See DxInquireCadCamCase –function.

If call was successful, Romexis client had launched PlanCAD program using the the given case, or in “new case” mode.

### Get Image Diagnose

Implemented only for Romexis. A new function in 2.6.2 specification (PmBridge library 4.5.0 or greater)

wchar_t wszText[20480];

extern "C" short  DxGetImageDiagnose(

int nImageNumber, WCHAR wszText);

nImageNumber	Input: image external ID number

wszText		Output: pointer to text string of WCHAR/wchar_t type,           max 20480 UNICODE characters

### Set Image Diagnose

Implemented only for Romexis. A new function in 2.6.2 specification (PmBridge library 4.5.0 or greater)

wchar_t wszText[20480];

extern "C" short  DxSetImageDiagnose(

int nImageNumber, WCHAR wszText);

Replaces image diagnose text with the given text. Use DxGetImageDianose –function first to get the existing diagnose, and add new text if necessary.

nImageNumber	Input: image external ID number

wszText		Input: new diagnose text, a pointer to text string of WCHAR/wchar_t type,  max 20480 UNICODE characters

## User and Patient Related Functions

### Adding a new patient

Of course, when a new patient is added to the practice management database the corresponding information needs to be passed to ROMEXIS as well. To do that:

err = DxInsertPatient (

const char *pszPatientID,

const char *pszFamilyName,

const char *pszFirstName,

const char *pszBirthDate,

const char *pszDoctor);

or (only for Romexis)

extern "C" short  DxInsertPatient_Ex (

const char *pszPatientID,

const char *pszFamilyName,

const char *pszFirstName,

const char *pszBirthDate,

const char *pszDoctor,

const char *pszGender

);

pszGender  “M”,”F”, or “O” (other)

Note! UNICODE compatible functions end with “W” -character.

### Querying patient information

To query patient’s name and birthdate in the ROMEXIS database, use the following function:

short DxGetPatientInfoW (

const wchar_t *patientID, // In

wchar_t* firstName, // Out

wchar_t* familyName, // Out

wchar_t* birthDate); // Out

First parameter is the ID of the patient you want to query. Three last string parameters are used to receive data back. You must provide at least 100 characters of space in the name parameters and 9 characters in the birthdate parameter. Birthdate is returned as a string, in format YYYYMMDD (e.g. “19991231”).

If patient name and birthday are known you can also query for the matching patient IDs calling two separate functions.

To retrieve the number of patients matching the query call

short  DxGetPatientIDWCount(

const wchar_t *firstName, // In

const wchar_t *lastName,  // In

const wchar_t *birthdate, // In

int *nPatients // Out: number of patients matching the search criteria

);

If the returned value is zero then no patients matching the criteria was found. Otherwise the matching IDs can be queried using the following function:

short  DxGetPatientIDW(

const wchar_t *firstName, // In

const wchar_t *lastName,  // In

const wchar_t *birthdate, // In

int maxPatients, // In: number of max patient ids to store in the list

wchar_t **patientIDs  // Out: patient ids

);

The caller is responsible for allocating memory to store the patient IDs. “patientIDs” is an array of arrays. The size of the array must be equal to or larger than maxPatients and each allocated sub-array must be at least 65 characters. Check return value of the function for possible error messages.

);

### Modifying patient information

When patient information is modified in the practice management system, (name changes due to marriage etc.) the corresponding information can be passed to ROMEXIS:

err = DxUpdatePatient (

const char *pszPatientID,

const char *pszFamilyName,

const char *pszFirstName,

const char *pszBirthDate,

const char *pszDoctor);

or (only for Romexis)

err = DxUpdatePatient_Ex (

const char *pszPatientID,

const char *pszFamilyName,

const char *pszFirstName,

const char *pszBirthDate,

const char *pszDoctor,

const char pszGender);

NOTE1! It is possible update only data of active patient, therefore DxSelectPatient –function call is required before calling DxUpdatePatient(_ex) function

NOTE2! It might be that patient data update is not allowed for logged in user.

NOTE3! It is possible to change patient ID for the current patient if it is allowed optionally in Romexis Admin module.

### Changing patient ID

Patient ID can be changed with the following function:

err = DxChangePatientID (

const char *pszOldPatientID,

const char *pszNewPatientID);

or

err = DxChangePatientIDW (

const wchar_t *pszOldPatientID,

const wchar_t *pszNewPatientID);

NOTE1! ID is changed only if there exists only one patient with the old ID and no patient exists with the new ID.

NOTE2! It might be that patient data update is not allowed for logged in user.

### Removing patient

Unneeded patients can also be removed from the ROMEXIS database via PMBRIDGE, but only if the patient don't have any images in the database:

err = DxDeletePatient (patientID);

### Deselection of the current patient

The current patient can be deselected by calling DxDeselectPatient().

### Setting ROMEXIS User

The ROMEXIS user can be set by calling

DxSetUser (

char *pszUserName,

unsigned short nAccessLevel

);

pszUserName will be used as the default for the user name which will be saved with the new images

nAccessLevel can be used to limit the rights of the user. It is possible to define if the user can acquire images, delete images or do administrative tasks (like change the settings) in Romexis . The access level parameters are defined in “PmBridge.h”. The lowest access level allows the user to view images. The access level parameters are used as a bit mask; so they will be added together. For example if the user is allowed to make exposures and delete images, the access level is

unsigned short nAccessLevel = DX_ACL_ACQUIRE_IMAGE +

DX_ACL_DELETE_IMAGE ;

## Study Related Functions

After a patient is selected in Romexis Client, following functions can be called. All functions will return IA_OK value if function call was successful.

### Get Number of Studies

Get numbers of studies for selected and active patient.

extern "C" short  CALLINGCONVENTION DxGetNumberOfStudies(short *pnStudyCount)

pnStudyCount result number of array of study numbers of active patient

### Get Study IDs

Get study ID numbers for selected and active patient as an array.

extern "C" short  CALLINGCONVENTION DxGetStudies(

int  arrStudies[],

int nArrLen

)

arrStudies result array of study numbers of active patient

nArrLen 	array size of  studies

### Get Study IDs and ‘Names’

Get study ID numbers and ‘work names’ of studies for selected and active patient as arrays. (An ‘enhanced’ function of DxGetStudies)

extern "C" short  CALLINGCONVENTION DxGetStudies_Name(

int  arrStudies[],

char **arrName ,

int nArrLen

)

arrStudies result array of study numbers of active patient

arrName 	result array of study ‘work names’ for studies

nArrLen 	array size of  studies

### Get Number of Images in Study

Fetch information of given study to arrays given as parameters. Number of images is saved in nImages parameter. The returned value indicates the max amount of images the given study can contain. To get further information about the actual images, use DxGetStudyInfo method.

short CALLINGCONVENTION DxGetNumberOfImagesInStudy(

long studyID, short &nImages);

### Get Study Info

Fetch information of given study to arrays given as parameters

short DxGetStudyInfo(long studyID, long *imageIDs, ImageRect *imageRects);

studyID	Study ID of selected and active patient

imageIDs	ImageID's of images within the study are stored here. Array should be allocated the same size as there are images (nImages). Number of images can be determined with DxGetNumberOfImagesInStudy function. ID has a value of 0 if no image data is associated with associated ImageRect.

imageRects	Image locations within a study are stored here as ImageRect data structures. The size of the array of should be of size nImages. Images are laid out in a 1920x1080 size canvas.

### Start Study Capture

Start study capture using given study ID number of existing study

of active patient

extern "C" short  CALLINGCONVENTION DxCaptureStudy(int nStudyID);

nStudyID study number in Romexis for selected and active patient

# PmBridge / Romexis Patient Information Fields

## PatientID

PatientID can be any string of a maximum length of 64 characters.

## FamilyName

FamilyName can be any string of a maximum length of 64 characters.

## FirstName

FirstName can be any string of a maximum length of 64 characters.

## Birthdate

Birthdate should be a string of 8 numbers formatted as YYYYMMDD. However, also formats YYYY*MM*DD and DD*MM*YYYY are recognized and converted to the right format.

## Doctor(optional)

Doctor can be any string of a maximum length of 64 characters.

# Best practices

## Opening patients

It is recommended to use the following PMBRIDGE command sequence regarding the opening of patients in ROMEXIS (see DxStart for more info):

Select Patient

-> If Patient not found, try to insert patient

-> If Patient found, try to update patient

## High Level Bridge Command Line Application

Some integrators have complained that PMBRIDGE is a bit complicated to use. Therefore a higher level bridge, DxStartW, has been developed by Planmeca. DxStartW encapsulates the complexities of PMBRIDGE, but the functionality is limited. See document “DxStart.doc” for more details.

## Using DxStart() Function

It is advisable to call the DxStart() function always before interacting with ROMEXIS i.e. call it not only at the beginning of the session but also when the patient is selected and you want to ROMEXIS to display the radiographs. This is not absolutely necessary but is good practice as it ensures that the ROMEXIS is running (the user may have quitted the application since you started it). Calling DxStart() causes minimal overhead if the ROMEXIS application is already running.

# Installation

## Windows Operating System

See the ROMEXIS installation manual for up to date installation instructions for ROMEXIS PMBRIDGE.

Please make sure that you only have one copy of PmBridge.DLL in the PC! The DLL must be located in the following ROMEXIS directory: 

C:\Program Files\Planmeca\Romexis\Pmbridge\Program

You should not copy PmBrige.DLL to your own directory because the dependency with the windows registry will then be broken.

You can obtain the path to ROMEXIS PMBRIDGE directory from the registry under the following key:

[HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\Windows\CurrentVersion\App Paths\Dimaxis.exe]

The path is of form

"Path"="C:\\Program Files\\Planmeca\\Romexis\\pmbridge"

Thus the subdirectory "Program" must be added to the string:

RegQueryValueEx (..., pszDimaxisPath, ...);

GetCurrentDirectory (sizeof(szCurrDir), szCurrDir);

SetCurrentDirectory (pszDimaxisPath);

SetCurrentDirectory ("Program");

HMODULE hPmBridge = LoadLibrary ( pszPath );

SetCurrentDirectory (szCurrDir);

Note that all the PMBRIDGE functions use calling convention _stdcall. Thus also programs written in other languages than C/C++ can use the library (if they are able to use C style NULL-terminated strings). An early preliminary version of the library used _cdecl calling conversion. This caused compatibility problems for the early PMBRIDGE interface users.

## Macintosh Operating System

The PMBRIDGE library file pmbridge.dylib is installed to the folder usr/lib.

The library calls the script Romexis.sh which is located in the following folder

/Applications/Planmeca/Romexis/pmbridge

The Romexis.sh starts up the ROMEXIS application.

## Conflicts with DIMAXIS PMBRIDGE

The pmbridge.dll file that is provided on the ROMEXIS installation CD is only meant to be used with ROMEXIS imaging application.

In the basic setup, Romexis and Dimaxis cannot be used both at the same time with PMBRIDGE.

While installing ROMEXIS, the PMBRIDGE registry information that points to the Dimaxis directory is changed to point to the ROMEXIS pmbridge.dll directory.

In order to use DIMAXIS one has to change the registry information to point back to 
the DIMAXIS pmbridge.dll directory.

# Other

## Example Program DxClientW

There is an example program DxClientW available for Windows (VS2015) and Mac OS X (XCode) for  clarifying the usage of PMBRIDGE. The example Windows sample uses a wrapper class CPmbWrap that dynamically loads the library in the constructor and frees it in the destructor.

## Unicode Compatibility

PMBRIDGE library 3.0.0 or greater is Unicode compatible. Unicode compatible versions of DxInsertPatient() and DxUpdatePatient() are DxInsertPatientW()…/W_ext()  and DxUpdatePatientW()/…W/ext().

Note! If Unicode compatible functions are used, then PatientID and BirthDate variables must be ANSI strings. All other must be UNICODE strings.

However, the calling application doesn’t have to be Unicode compatible to use PMBRIDGE.

## Log File

PmBridge Romexis library creates a log file “PmBridge_<timestamp>.log” in %temp% folder on Windows. On Mac OS X log file is created in user’s TEMP environment folder.

## Clinic Status XML Sample

<?xml version="1.0" ?>

<!--DO NOT EDIT THIS FILE!-->

<clinics count="3 " version="4.5.0.73.B">

<summary>

<dentalunits online="0" errors="0" warnings="0" total="669" >

</dentalunits>

<xrays online=”0" errors="0" warnings="0" total="11" >

</xrays>

<mills online="1" errors="1" warnings="0" total="17 " >

</mills>

<errorunits>

<unit name ="PlanMill_40_demo"  type="PlanMill 40" error="400.3" </unit>

</errorunits>

</summary>

</clinics>

## Real Basic PmBridge Interface Declarations Samples

C++ WCHAR  =>  WString type

C++ char  => CString  type

Function For Windows DLL

Declare Function DxStart Lib "pmbridge.dll" () As Integer

Declare Function DxSelectPatient Lib "pmbridge.dll" (ByVal PatientID As CString, ByRef ImageCnt As Integer) As Integer

Declare Function DxInsertPatientW Lib "pmbridge.dll" (ByVal PatientID As CString,ByVal Family As WString,ByVal First As WString ,ByVal BirthDate As CString, ByVal Doctor As WString) As Integer

etc.